# AURORA-6: Preliminary Sun-Synchronous Constellation Mission Design

Six-satellite / three-plane concept for <=24 h access to 30-60 deg N under an explicit preliminary access assumption

Prepared 2026-09-27 | Status: preliminary, evidence-gated

# 1. Requirement and interpretation

Design target: a six-satellite constellation in circular sun-synchronous orbit (SSO), providing a preliminary maximum revisit gap below 24 hours to a target latitude band of 30 to 60 deg N. Because the prompt does not specify sensor field of view, the access requirement is explicitly modeled as a 4 deg Earth-surface central-angle half-swath. This is a generous first-order access criterion, not a claim about a specific sensor.

ASSUMPTION: The mission is allowed to use three Falcon 9 rideshare launches, with two 25 kg spacecraft per launch/plane.

CALCULATED: At 550 km circular altitude, J2 secular nodal precession requires inclination about 97.59295 deg for a tropical-year-equivalent solar precession target of 0.9856473 deg/day.

PENDING: Mission-specific launch trajectory, atmospheric density/drag environment, detailed thermal model, and real sensor-derived swath.

# 2. Baseline orbital elements

| Parameter | Baseline | Basis |
| --- | --- | --- |
| Altitude | 550 km | Design choice; balances revisit, drag, thermal environment, and deorbit practicality. |
| Semimajor axis | 6928.137 km | WGS84 equatorial radius 6378.137 km + 550 km. |
| Eccentricity | 0 | Circular-orbit concept. |
| Inclination | 97.59295 deg | Solved from secular J2 nodal precession equation. |
| RAAN plane 1 | 0 deg at design epoch | Reference orientation. |
| RAAN plane 2 | 120 deg at design epoch | Walker-like plane spacing. |
| RAAN plane 3 | 240 deg at design epoch | Walker-like plane spacing. |
| Argument of latitude, plane 1 | 0 deg, 180 deg | Two spacecraft 180 deg apart. |
| Argument of latitude, plane 2 | 120 deg, 300 deg | Walker F=2 phasing. |
| Argument of latitude, plane 3 | 240 deg, 60 deg | Walker F=2 phasing. |

# 3. Orbital mechanics closure

Using mu = 398600.4418 km^3/s^2, Re = 6378.137 km, J2 = 1.08262668e-3, e = 0, and the standard secular nodal-precession equation:

Omega_dot = -3 J2 n Re^2 cos(i) / [2 a^2 (1-e^2)^2]

The computed solution is i = 97.59295 deg. The resulting orbital period is 95.64988 min. These values are internally consistent with the stated semimajor axis and J2 model.

CROSS-CHECK: NASA publishes a 705 km, 98.2 deg SSO example with about 0.986 deg/day nodal precession; the AURORA-6 550 km solution is different because altitude is different, not because the SSO relationship was ignored.

# 4. Revisit / access analysis

Numerical model: six circular spacecraft, three planes, Walker-like F=2 phasing, Earth rotation, secular RAAN drift, spherical Earth access, 1-minute time steps, and a 0.5 deg latitude x 5 deg longitude grid over 30-60 deg N for an 8-day run. A cyclic gap check was applied so the end-of-window to start-of-window interval is not silently omitted.

| Test | Result | Status |
| --- | --- | --- |
| Grid points | 4392 | CALCULATED |
| Time horizon | 8 days | CALCULATED |
| Time step | 1 min | CALCULATED |
| Access half-swath | 4 deg central angle | ASSUMPTION |
| All points accessed | Yes | CALCULATED |
| Maximum cyclic access gap | 13.17 h | CALCULATED |
| Requirement | <=24 h | PASS under stated assumptions |
| Coarse 16-day check (1 deg x 10 deg grid) | 13.87 h max cyclic gap | CALCULATED, sensitivity check |

The revisit result is not a proof for a real sensor. The dominant unresolved variable is sensor look angle / true ground swath. Re-run the access model using the actual payload field of regard, elevation mask, Earth ellipsoid, attitude law, terrain/elevation, cloud constraints, and mission-specific imaging rules before flight acceptance.

# 5. Plane-spacing and launch-phasing logic

The nominal SSO precession rate is about 0.9856473 deg/day. A 120 deg RAAN separation therefore corresponds to 120 / 0.9856473 = 121.747 days, or about 4.01 months. This is close to SpaceX current public language that SSO missions occur approximately every 4 months. The coincidence is useful but is not a booked launch schedule.

| Element | Nominal plan | Closure status |
| --- | --- | --- |
| Launch site | Vandenberg, California | Consistent with high-inclination / SSO access; exact mission integration pending. |
| Launch vehicle | Falcon 9 Rideshare | Real current program. |
| Payload per launch | 2 x 25 kg = 50 kg | Matches current 50 kg-to-SSO rideshare baseline. |
| Launch 1 | Plane 1 target RAAN 0 deg | Nominal reference. |
| Launch 2 | 121.747 days later, plane 2 target RAAN 120 deg | Phasing calculation; exact launch date/time pending. |
| Launch 3 | 243.495 days after launch 1, plane 3 target RAAN 240 deg | Phasing calculation; exact launch date/time pending. |
| LTAN | 10:30 local solar time, +/-15 min | Mission assumption modeled after common Earth-observation practice; exact launch window pending. |

IMPORTANT: A launch "window" is not fully closed by this document. The exact clock time, azimuth, insertion state, and provider-specific performance at 550 km require the launch provider mission design and a current flight slot. The artifact therefore records the phasing window mathematically but does not fabricate a booked launch date.

# 6. Delta-v budget

The constellation avoids large onboard plane changes by creating plane spacing through launch geometry and timing. A 10 deg plane change at 550 km would require about 1322 m/s, which is approximately 5.5 times the entire nominal maneuver budget below. Therefore plane change is a prohibited baseline operation.

| Stage | Delta-v | Basis / status |
| --- | --- | --- |
| Insertion / commissioning trim | 15 m/s | Design allocation; actual launch-injection dispersions pending. |
| Plane change | 0 m/s nominal | Architecture choice; plane changes performed by launch geometry, not spacecraft propulsion. |
| Station-keeping | 125 m/s over 5 years | Design allocation, not a density-model prediction. |
| End-of-life disposal | 70.01 m/s | Calculated one-burn apogee burn from 550 km circular to an ellipse with 300 km perigee; drag completes disposal. |
| Contingency | 30 m/s | Design reserve. |
| TOTAL | 240.01 m/s | Closed allocation. |

Propellant cross-check assumption: If a representative monopropellant system with Isp = 220 s is assumed, a 240.01 m/s total budget requires a propellant fraction of approximately 10.55 percent by the Tsiolkovsky rocket equation. For a 25.0 kg wet spacecraft this is approximately 2.64 kg of propellant. This is a sizing assumption, not a selected flight component.

# 7. End-of-life calculation

For a single retrograde burn at 550 km circular apogee, the transfer ellipse is defined by rp = Re + 300 km and ra = Re + 550 km. The calculated burn magnitude is 70.01 m/s. NASA notes that SmallSats above roughly 500 km cannot be assumed to naturally deorbit quickly under all atmospheric conditions, so a controlled disposal maneuver is retained in the design.

# 8. Power budget

| Mode | Average load | Time per orbit (worst-case geometry) | Energy/orbit |
| --- | --- | --- | --- |
| Sunlit | 100 W | 60.038 min | 100.06 Wh |
| Eclipse | 70 W | 35.611 min | 41.55 Wh |
| Total | - | 95.650 min | 141.61 Wh |

With an assumed 85 percent power-path efficiency, the average required array power while sunlit is about 166.49 W. A 1.0 m^2 array using a 400 W/m^2 BOL reference point and an assumed 25 percent combined geometry/degradation derating gives about 300 W EOL at reference incidence, leaving substantial first-order margin.

Battery: A minimum sizing result of about 75.0 Wh follows from 41.55 Wh eclipse energy, 80 percent allowable depth of discharge, 90 percent battery efficiency, and 30 percent design margin. A 90 Wh installed target is used here to preserve margin.

# 9. Thermal first-order sizing

Assume 100 W average internal dissipation during the powered thermal case, 25 W of external absorbed heat not already counted in the internal dissipation, 20 percent thermal margin, emissivity 0.90, and 293 K radiator temperature. Stefan-Boltzmann radiation then gives an effective radiator area of approximately 0.399 m^2, rounded to 0.40 m^2.

WEAK / PENDING: The thermal model omits a full time-dependent spacecraft attitude, Earth IR view factor, albedo geometry, hot/cold operational cases, internal equipment temperature limits, MLI details, and detailed radiator coating data. This is a first-pass sizing calculation, not a thermal-vacuum qualification.

# 10. Explicit assumptions and uncertainties

| Item | Type | Why it matters |
| --- | --- | --- |
| 4 deg access half-swath | Assumption | Dominant revisit driver; must be replaced by real sensor geometry. |
| 550 km altitude | Design choice | Changes period, SSO inclination, drag, and access. |
| 10:30 LTAN +/-15 min | Assumption | Sets illumination regime; exact mission LTAN may differ. |
| 25 kg spacecraft | Assumption | Used for propellant sizing and launch accommodation. |
| 220 s Isp | Assumption | Representative propulsion sizing only. |
| 125 m/s station-keeping / 5 yr | Allocation | Not a validated atmospheric-drag prediction. |
| 25% solar derating | Assumption | Lumps angle, degradation, temperature, and packaging effects. |
| Thermal external heat load 25 W | Assumption | Must be replaced by full environmental model. |
| Falcon 9 rideshare 50 kg to SSO | Published program fact | Confirms target class and mass baseline, but not a mission-specific 550 km performance curve. |

# 11. Failure conditions

If actual sensor swath is narrower enough that the 24 h access requirement fails, constellation size/altitude must be redesigned.

If launch-provider insertion performance or interface limits invalidate the two-satellite-per-launch concept, the launch architecture changes.

If drag modeling shows the 125 m/s allocation is inadequate for the required lifetime, propulsion or altitude must change.

If full thermal analysis cannot reject the actual worst-case heat load within equipment limits, the thermal design fails.

If any later model changes an input without updating dependent numbers, the artifact is internally inconsistent and must be re-run.

# Source ledger

| ID | Authority | URL | Use |
| --- | --- | --- | --- |
| S1 | NGA WGS84 | https://earth-info.nga.mil/?action=wgs84&dir=wgs84 | Earth semimajor axis and GM. |
| S2 | NASA Earth Fact Sheet | https://nssdc.gsfc.nasa.gov/planetary/factsheet/earthfact.html | Earth constants, solar irradiance, albedo, J2 context. |
| S3 | NASA Sun-Synchronous Orbit example | https://ntrs.nasa.gov/api/citations/20205004141/downloads/Introduction%20to%20Orbitalaft%20Engineers%20CHARTS%20REV%20A.pdf | J2 nodal precession equation and SSO example. |
| S4 | NASA SmallSat Power, 2026 | https://www.nasa.gov/smallsat-institute/sst-soa/power-subsystems/ | Solar-cell BOL/EOL and W/m^2 data. |
| S5 | NASA Deorbit Systems, 2026 | https://www.nasa.gov/smallsat-institute/sst-soa/deorbit-systems/ | LEO lifetime and deorbit context. |
| S6 | SpaceX Rideshare | https://new.spacex.com/rideshare | Current public 50 kg to SSO baseline and approximately 4-month SSO cadence. |
| S7 | SpaceX Falcon 9 User Guide, 2025 | https://www.spacex.com/assets/media/falcon-users-guide-2025-05-09.pdf | Falcon 9 mission/SSO launch-service context. |
| S8 | SpaceX Falcon 9 vehicle | https://new.spacex.com/vehicles/falcon-9 | Current vehicle and payload-to-LEO overview. |